Deep Dive: RS485 Multi-Drop Feasibility Analysis

Architecture Overview

The stock anchor firmware has a clean 4-layer separation for the host link:

UCI Backend (precompiled uwb-stack lib — cannot modify)

↓ ↑

UCI Transport (uci_transport.c — raw UCI framing, no SLIP/CRC)

↓ ↑

Reporter / USB_UART TX (usb_reporter.c → usb_uart_tx.c → flush_report_buf)

↓ ↑

HAL UART (HAL_uart.c — 115200 8N1, ISR-driven RX)

Three Clean Interception Points for Custom Protocol

You can insert a custom framing layer without modifying the precompiled uwb-stack:

Layer	File	What you do

TX wrapper	usb_reporter.c	Replace reporter_instance with your own — prepend [NodeID][PayloadLen][UCI...] before port_tx_msg()

RX wrapper	task_uci.c uci_notify_data_in()	Parse your header, discard non-matching packets, then forward stripped UCI bytes → uci_tp_read()

PHY layer	HAL_uart.c	Add RS485 TX-enable GPIO toggle in deca_uart_transmit() + deca_uart_init()

Stock Data Flow (for reference)

RX entry:

UART ISR → deca_uart_receive() → byte into cc_buff

→ calls uart_rx_callback (which = NotifyControlTaskData)

→ uci_notify_data_in() → queue → uci_task_process_incoming()

→ uci_tp_read() → raw UCI 4-byte header + payload → UCI core

TX entry:

UCI backend builds response → reporter_instance.print()

→ usb_print() → port_tx_msg() → copy_tx_msg() into circular buffer

→ FlushTask (5ms loop) calls flush_report_buf()

→ deca_uart_transmit() → UART TX

What You Need to Change on Each Anchor

1. Assign a unique Node ID — add to custom_board.h or driver_app_config.c:

#define RS485_NODE_ID  0x01  // different per anchor: 0x01, 0x02, 0x03

2. Custom TX in usb_reporter.c — wrap outgoing UCI bytes:

static error_e rs485_print(char *buff, int len) {

uint8_t frame[4 + 2 + len];  // [NodeID(1)][Reserved(1)][PayloadLen(2)][UCI...]

frame[0] = RS485_NODE_ID;    // source node

frame[1] = 0x00;             // flags (poll bit? timestamp present?)

frame[2] = (len >> 8) & 0xFF; // payload length big-endian

frame[3] = len & 0xFF;

memcpy(&frame[4], buff, len);

return port_tx_msg(frame, sizeof(frame));

}

To add timestamping, insert a 4-byte qrtc_get_us() microsecond timestamp at frame[4..7] (adjust offsets).

3. Custom RX in task_uci.c uci_notify_data_in() — strip and filter:

void uci_notify_data_in(usb_data_e res, void *data) {

struct cc_buff *buf = (struct cc_buff *)data;

// First byte = target NodeID (0x00 = broadcast to all)

uint8_t target_node;

cc_buff_copy_n(buf, &target_node, 1);  // consume target NodeID

if (target_node != RS485_NODE_ID && target_node != 0x00) {

return;  // discard — not for this anchor

}

// Strip the rest of your header, then forward remaining UCI bytes

// (you may need a temporary cc_buff for the stripped payload)

struct uci_mail_data msg = {.message_type = UCI_DATA_IN, .buf = buf};

qmsg_queue_put(uci_msg_queue, &msg);

}

4. RS485 PHY in HAL_uart.c — add TX-enable GPIO control:

void deca_uart_init(CommRxCallback callback) {

nrf_gpio_cfg_output(RS485_DE_RE_PIN);

nrf_gpio_pin_clear(RS485_DE_RE_PIN);  // RX mode initially

// ... rest of stock init ...

}

int deca_uart_transmit(uint8_t *ptr, uint16_t size) {

nrf_gpio_pin_set(RS485_DE_RE_PIN);   // enable driver (TX)

int ret = NRF_SUCCESS;

for (int i = 0; i < size; i++) {

if (app_uart_put(ptr[i]) != NRF_SUCCESS) { ret = _ERR_UART_TX; break; }

}

// Wait for TX shift register empty, then disable driver

while (!app_uart_tx_complete());     // or use nrf_uart_event_clear + wait

nrf_gpio_pin_clear(RS485_DE_RE_PIN); // back to RX mode

return ret;

}

Note: The nRF52833 on DWM3001CDK has 2 UART instances. You could keep UART0 for RS485 and use UART1 for debugging, or just use the same UART0 with RS485 PHY.

Important Limitation (Precompiled uwb-stack)

The uwb-stack is delivered as precompiled libraries in Libs/uwbstack_libs/delivery/full/Release/lib/. You cannot modify how it builds RANGE_DATA_NTF or processes UCI commands.

What this means for your design:

- ✅ Add Node ID / length framing around UCI — yes, at the reporter layer

- ✅ Add timestamp — yes, insert into your custom header on TX

- ✅ Poll-response scheme — yes, master addresses a specific anchor by NodeID; only that anchor responds

- ❌ Modify UCI payload format — no, must remain standard FiRa UCI

- ❌ Add custom GID/OID — possible only if the uwb-stack already supports it (vendor GIDs 0x0B–0x0F exist but are build-dependent)

Master (ESP32) Protocol Design

Your RS485 frame on the wire could look like:

[TargetNodeID (1)] [SourceNodeID (1)] [Flags (1)] [Timestamp (4, μs)] [PayloadLen (2)] [UCI Frame...]

For polling:

1. Master sends with TargetNodeID=0x01, SourceNodeID=0x00 (master), UCI command

2. Only Anchor 0x01 processes it, sends response with SourceNodeID=0x01

3. Master parses response, routes to correct UciSessionManager instance by NodeID

For continuous ranging results from all 3 anchors:

- Option A: Sequential polling — master sends SESSION_START to one anchor, receives RANGE_DATA_NTF every ~120ms, then stops and moves to next anchor. Cycle = 3 × 120ms = 360ms.

- Option B: All anchors range simultaneously with the same phone (multicast mode using MULTI_NODE_MODE=1), each sends RANGE_DATA_NTF. The ESP32 demuxes by mac_address @ payload[25:26] mapped to NodeID. This is more efficient.

Bottom Line

Yes, the SDK architecture cleanly allows you to insert a custom protocol layer for RS485 multi-drop. The stock code has well-defined interception points at reporter_instance.print() (TX) and uci_notify_data_in() (RX) where you can wrap/strip framing without touching the UCI core or precompiled uwb-stack.